ECA5610- 5g/6g communication

Slot A

V.Rushmitha

192512050

Experiment 4: OFDMA Resource Block Allocation

AIM:

To simulate and analyze OFDMA (Orthogonal Frequency Division Multiple Access)

resource block allocation and evaluate system performance in terms of resource utilization and

user throughput using MATLAB.

Objective 1:

To determine the Number of Resource Blocks (RBs) allocated in an OFDMA system

based on available bandwidth and system configuration.

Matlab Code:

clc; clear; close all;

% System parameters

bandwidth = 10e6; % 10 MHz

rb_size = 180e3; % 180 kHz per RB

% Total Resource Blocks

total_RB = floor(bandwidth / rb_size);

% Users

users = 1:6;

% RB allocation (simple equal + remainder distribution)

base = floor(total_RB / length(users));

alloc = base * ones(1,length(users));

alloc(1:mod(total_RB,length(users))) = alloc(1:mod(total_RB,length(users))) + 1;

utilization = (alloc / total_RB) * 100;

disp('Total Resource Blocks:')

disp(total_RB)

disp('RB Allocation per User:')

disp(alloc)

figure

% -------- Graph 1: RB Allocation per User --------

subplot(2,1,1)

stem(users, alloc, 'filled', 'LineWidth', 2)

title('OFDMA Resource Block Allocation per User')

xlabel('User Index')

ylabel('Number of Resource Blocks')

grid on

% -------- Graph 2: RB Utilization --------

subplot(2,1,2)

plot(users, utilization, '-o', 'LineWidth', 2)

title('OFDMA Resource Block Utilization (%)')

xlabel('User Index')

ylabel('Utilization (%)')

grid on

output:

Result:

The first graph shows how available resource blocks are distributed among multiple users in the

OFDMA system.

The second graph represents utilization efficiency, indicating how effectively the spectrum is

shared among users.

Objective 2:

To evaluate the Resource Block Utilization (%) by analyzing how efficiently available

RBs are assigned to active users.

Matlab Code:

clc; clear; close all;

% Total system resource blocks

total_RB = 50;

% Number of active users

users = 1:5;

% RB allocation to users (example scenario)

alloc = [12 10 9 8 11];

% Resource Block Utilization (%)

util = (alloc / total_RB) * 100;

disp('Resource Block Utilization (%) per User:')

disp(util')

figure

% -------- Graph 1: RB Allocation --------

subplot(2,1,1)

plot(users, alloc, '-o', 'LineWidth', 2)

title('OFDMA Resource Block Allocation per User')

xlabel('User Index')

ylabel('Number of Resource Blocks')

grid on

% -------- Graph 2: Utilization (%) --------

subplot(2,1,2)

plot(users, util, '-s', 'LineWidth', 2)

title('OFDMA Resource Block Utilization (%)')

xlabel('User Index')

ylabel('Utilization (%)')

grid on

output:

Result:

The first graph shows how resource blocks are distributed among multiple users in the OFDMA

system.

The second graph indicates the efficiency of spectrum usage by each user in terms of percentage

utilization.

Objective 3:

To calculate the User Throughput (Mbps) based on allocated resource blocks and

modulation efficiency in the OFDMA system.

Matlab Code:

clc; clear; close all;

% System parameters

rb_bw = 180e3; % Bandwidth per RB (180 kHz)

users = 1:5;

% Allocated Resource Blocks per user

alloc_RB = [12 10 9 8 11];

% Modulation efficiency (bits/symbol)

mod_eff = [2 4 6 4 2]; % QPSK, 16QAM, 64QAM etc.

% Throughput calculation (Mbps)

throughput = (alloc_RB .* rb_bw .* mod_eff) / 1e6;

disp('User Throughput (Mbps):')

disp(throughput')

figure

% -------- Graph 1: Throughput per User --------

subplot(2,1,1)

plot(users, throughput, '-o', 'LineWidth', 2)

title('OFDMA User Throughput')

xlabel('User Index')

ylabel('Throughput (Mbps)')

grid on

% -------- Graph 2: RB vs Throughput --------

subplot(2,1,2)

plot(alloc_RB, throughput, '-s', 'LineWidth', 2)

title('Resource Blocks vs Throughput Relationship')

xlabel('Allocated Resource Blocks')

ylabel('Throughput (Mbps)')

grid on

output:

Result:

1. The Resource Block (RB) allocation was successfully simulated for multiple users in the

OFDMA system, demonstrating how available spectrum is distributed among active

users.

2. The Resource Block Utilization (%) analysis showed efficient usage of system resources,

indicating how effectively the available RBs are assigned to users.